Micro-ROS on Arduino Nano ESP32

Complete Step-by-Step Setup Guide

Windows 11 | Arduino IDE | USB Serial Transport

February 2026 - Zero-knowledge edition

# Table of Contents

(Update fields in Word: right-click the TOC and choose Update Field.)

# 1. What You Need Before Starting

## 1.1 Hardware Checklist

| Item | Details | Status |
| --- | --- | --- |
| Arduino Nano ESP32 | The board itself (ESP32-S3 chip, USB-C connector) | Required |
| USB-C data cable | Must be a DATA cable, not charge-only. Your new cable should work. | Required |
| Windows 11 PC | Any Windows 11 machine with a free USB port | Required |
| Internet connection | Needed to download Arduino IDE, board packages, and libraries | Required |

## 1.2 What You Will Install (In Order)

- Arduino IDE 2.x - The program you use to write, compile, and upload code to the board.

- ESP32 Board Package - Tells Arduino IDE how to compile code for your specific chip (ESP32-S3).

- micro_ros_arduino Library - The micro-ROS library that lets your ESP32 talk to ROS 2.

- Python fix (if needed) - The ESP32 build tools use Python scripts internally. Sometimes needs a quick fix on Windows.

## 1.3 How This Will All Connect Later

Right now we are only setting up the ESP32 side. Here is the big picture so you understand where this fits:

- Arduino Nano ESP32 (what we are setting up now): Runs micro-ROS firmware. Controls stepper motors. Talks over USB serial.

- Jetson Orin Nano (set up later): Runs Ubuntu + ROS 2 Humble + micro-ROS agent. The 'brain' that sends commands.

- USB cable between them: Carries micro-ROS messages (using a protocol called XRCE-DDS) over serial.

Today's goal: Get a micro-ROS publisher example compiled, uploaded to the ESP32, and confirmed working via the Arduino IDE Serial Monitor.

# 2. Install Arduino IDE on Windows 11

## Step 2.1 - Download Arduino IDE

- Open your web browser (Edge, Chrome, etc.).

- Go to: https://www.arduino.cc/en/software

- You will see a page titled 'Software' with a big download button.

- Click 'WINDOWS' under 'Arduino IDE 2.x.x'. It will download a file like arduino-ide_2.3.4_Windows_64bit.exe

- If it asks 'Just download' or 'Contribute & Download', click 'Just download.'

- Wait for the download to finish. It is about 250 MB.

## Step 2.2 - Install Arduino IDE

- Double-click the downloaded .exe file to start the installer.

- If Windows asks 'Do you want to allow this app to make changes?' click 'Yes.'

- The installer will show a license agreement. Click 'I Agree.'

- It will ask who to install for. Choose 'All Users' (recommended). Click 'Next.'

- Leave the install location as default. Click 'Install.'

- Wait for installation to complete. This takes 1-2 minutes.

- When it says 'Completed,' click 'Finish.'

- Arduino IDE is now installed. You can find it in your Start Menu under 'Arduino IDE.'

## Step 2.3 - Open Arduino IDE for the First Time

- Click the Start Menu and type 'Arduino IDE'. Click it to open.

- On first launch, it may take 30-60 seconds to load. Be patient.

- You will see a blank editor window with some default code (gray text on white background).

- The IDE may ask to install some updates. Let them install if prompted.

Arduino IDE is now installed and open. Leave it open for the next steps.

# 3. Install the ESP32 Board Package

Arduino IDE does not know about ESP32 chips by default. You need to tell it where to find the ESP32 tools, then install them.

## Step 3.1 - Add the ESP32 Board URL

- In Arduino IDE, click File (top-left menu bar) > Preferences.

- A settings window will open.

- Find the field labeled 'Additional boards manager URLs.' It is near the bottom of the window.

- If the field is empty, paste this exact URL into it:

https://espressif.github.io/arduino-esp32/package_esp32_index.json

- If there is already a URL in that field, click the small icon to the right of the field (it looks like a tiny window). A box will open showing URLs one per line. Add the URL above on a new line.

- Click 'OK' to close the Preferences window.

## Step 3.2 - Install the ESP32 Board Package

- Click Tools > Board > Boards Manager...

- A panel will open on the left side. It may take a moment to load.

- In the search box at the top, type: esp32

- You will see 'esp32 by Espressif Systems.' This is the official package.

- Click 'Install' next to it. Choose version 2.0.17 if available, or the latest 2.x version. Do NOT use version 3.x as micro-ROS has known compatibility issues with it.

IMPORTANT: Use ESP32 board package version 2.0.x (not 3.x). The micro_ros_arduino library is tested against 2.0.x.

- The installation will download about 300-400 MB of tools. This can take 5-10 minutes depending on your internet speed.

- You will see a progress bar. Wait until it says 'INSTALLED' next to the package.

The ESP32 board package is now installed. Arduino IDE can now compile code for your Arduino Nano ESP32.

# 4. Connect Your Arduino Nano ESP32 to the PC

## Step 4.1 - Plug In the Board

- Take your Arduino Nano ESP32 board and your new USB-C data cable.

- Plug the USB-C end into the board, and the other end into your Windows 11 PC.

- The board should power on. Look for a small green LED on the board that lights up. This means it is receiving power.

- You may also see the RGB LED flash briefly (orange/blue). This is normal.

If no LED lights up at all, try a different USB port on your PC. Some front-panel USB ports do not provide enough power.

## Step 4.2 - Check Windows Device Manager

We need to verify Windows detected the board and assigned it a COM port.

- Press Win + X on your keyboard (hold the Windows key and press X).

- Click 'Device Manager' from the menu that appears.

- In Device Manager, look for 'Ports (COM & LPT).' Click the arrow to expand it.

- You should see an entry. What you see depends on the board's mode:

| What You See | What It Means | Action |
| --- | --- | --- |
| USB Serial Device (COMx) | Board is in normal mode. Ready to use. | Note the COM number |
| Arduino Nano ESP32 (COMx) | Board detected by name. Ready to use. | Note the COM number |
| Unknown device (yellow warning) | Driver not installed automatically. | See Step 4.3 below |
| Nothing new appears | Cable might be charge-only, or USB port issue. | Try different cable/port |

Write down your COM port number (e.g., COM3, COM5, COM7). You will need it later.

## Step 4.3 - If the Board Is Not Detected (Troubleshooting)

The Arduino Nano ESP32 uses the ESP32-S3's built-in USB (not an external chip). Windows 11 usually installs the driver automatically. If it doesn't:

- Try the boot mode trick: Double-press the RST (reset) button on the board quickly. The RGB LED should turn green. This puts the board in 'bootloader' mode which Windows may recognize differently.

- Try a different USB port. Use a port directly on your PC (back panel of a desktop), not through a USB hub.

- Try a different cable. Some USB cables only carry power, not data. Even new cables can be charge-only.

- Restart your PC with the board still plugged in. Sometimes Windows needs a reboot to pick up new USB devices.

## Step 4.4 - Select the Board in Arduino IDE

- In Arduino IDE, click Tools > Board > esp32 (you will see the esp32 submenu).

- Scroll down and select 'Arduino Nano ESP32.'

If you do not see 'Arduino Nano ESP32' in the list, the ESP32 board package (Section 3) was not installed correctly. Go back and repeat Section 3.

- Click Tools > Port > Select the COM port you noted in Step 4.2 (e.g., COM5).

Your board is now connected, recognized, and selected in Arduino IDE.

# 5. Test the Board with Blink (Sanity Check)

Before installing micro-ROS, let's verify everything works by uploading a simple LED blink program.

## Step 5.1 - Open the Blink Example

- Click File > Examples > 01.Basics > Blink

- A new window opens with code that blinks an LED.

## Step 5.2 - Compile (Verify)

- Click the checkmark button (Verify) in the top-left toolbar (or press Ctrl+R).

- The IDE will compile the code. You will see messages scrolling in the black output panel at the bottom.

- This first compile takes 1-3 minutes because it is building the entire ESP32 framework.

- Wait for 'Done compiling' to appear in the bottom status bar.

If you get Python errors like 'python: command not found' or 'ModuleNotFoundError: pyserial' - see Section 5.4 below.

## Step 5.3 - Upload to Board

- Click the right-arrow Upload button in the toolbar (or press Ctrl+U).

- The IDE will compile again and then flash the firmware to your board.

- You will see 'Connecting...' and then a progress bar.

- If it gets stuck on 'Connecting...': Double-press the RST button on the board while the IDE is trying to connect. This forces bootloader mode.

- When it says 'Done uploading,' look at your board. The built-in LED should be blinking on and off every second.

Your board is working. Arduino IDE can compile code and upload it to your Arduino Nano ESP32.

## Step 5.4 - Fix Python Errors (If Any)

The ESP32 build tools use Python scripts internally (for compiling, not for your code). On Windows, sometimes Python is not correctly linked. If you got Python errors during compile:

- Check if Python is installed: Open Command Prompt (press Win + R, type cmd, press Enter). Type python --version and press Enter.

- If it says 'Python 3.x.x' - Python is installed. Skip to step 4.

- If Python is NOT installed: Go to https://www.python.org/downloads/ > Download the latest Python 3.x > Run the installer > CHECK THE BOX 'Add python.exe to PATH' at the bottom of the installer > Click 'Install Now.'

- Install pyserial: In Command Prompt, type:

pip install pyserial

- Close and reopen Arduino IDE. Try compiling again.

# 6. Install the micro-ROS Arduino Library

This is the library that gives your ESP32 the ability to communicate with ROS 2. It is provided as a precompiled .zip file that you add to Arduino IDE.

## Step 6.1 - Download the Library

- Open your web browser.

- Go to: https://github.com/micro-ROS/micro_ros_arduino/releases

- You will see a list of releases. Each release is tagged with a ROS 2 version name.

- Find the latest release tagged 'humble' (this matches your Jetson's ROS 2 Humble).

- Under 'Assets,' click the .zip file to download it. It will be named something like micro_ros_arduino-humble-v2.x.x.zip. The file is about 30-40 MB.

- Save it somewhere you can find it (e.g., your Downloads folder). Do NOT unzip it.

Do NOT unzip the .zip file. Arduino IDE needs the .zip file directly.

## Step 6.2 - Add the Library to Arduino IDE

- In Arduino IDE, click Sketch > Include Library > Add .ZIP Library...

- A file browser window will open.

- Navigate to where you saved the .zip file (e.g., Downloads folder).

- Select the micro_ros_arduino .zip file and click 'Open.'

- Arduino IDE will extract and install the library. You should see a message like 'Library installed' in the bottom output panel.

## Step 6.3 - Critical Fix: Add ESP32-S3 Support

This step is required. The micro_ros_arduino library comes with precompiled binaries for the standard ESP32, but your Arduino Nano ESP32 uses the ESP32-S3 chip. You need to copy the ESP32 binaries and rename them for ESP32-S3.

- Open Windows File Explorer (press Win + E).

- Navigate to your Arduino libraries folder. The path is typically:

C:\Users\YOUR_USERNAME\Documents\Arduino\libraries\micro_ros_arduino\src

Replace YOUR_USERNAME with your actual Windows username. If you are unsure, open File Explorer, go to Documents > Arduino > libraries > micro_ros_arduino > src.

- Inside the src folder, you will see a folder named esp32. This contains the precompiled micro-ROS library files.

- Right-click the 'esp32' folder > Copy.

- Right-click in the same src folder > Paste.

- A new folder appears named 'esp32 - Copy.' Rename it to: esp32s3

The folder MUST be named exactly 'esp32s3' (all lowercase, no spaces). Not 'ESP32S3' or 'esp32-s3'. If named wrong, compilation will fail with 'micro_ros' not found errors.

- Verify: You should now have two folders inside src:

micro_ros_arduino/

src/

esp32/        (original - for standard ESP32)

esp32s3/      (your copy - for Arduino Nano ESP32)

The micro-ROS library is now installed and configured for your Arduino Nano ESP32.

# 7. Compile and Upload a micro-ROS Example

## Step 7.1 - Open the Publisher Example

- In Arduino IDE, click File > Examples.

- Scroll down in the Examples list until you find 'micro_ros_arduino.'

- Click it to expand, then click 'micro_ros_publisher.'

- A new sketch window opens with the micro-ROS publisher example code.

This example creates a ROS 2 node on the ESP32 that publishes an incrementing integer to a topic. It is the simplest micro-ROS test.

## Step 7.2 - Verify Board and Port Settings

Before compiling, double-check these settings:

- Tools > Board: Should say 'Arduino Nano ESP32'

- Tools > Port: Should show your COM port (e.g., COM5)

## Step 7.3 - Compile

- Click the checkmark Verify button.

- This will take 3-7 minutes on the first build. The ESP32 micro-ROS stack is large. Subsequent builds are faster.

- Watch the output panel at the bottom. You will see hundreds of lines scrolling by. This is normal.

- Wait for 'Done compiling' and look for the memory usage summary:

Sketch uses XXXXXX bytes (XX%) of program storage space.

Global variables use XXXXXX bytes (XX%) of dynamic memory.

If it fails with 'could not find micro_ros': verify you did Step 6.3 correctly (the esp32s3 folder copy).

If it fails with 'python not found': do Step 5.4 (install Python + pyserial).

## Step 7.4 - Upload

- Click the right-arrow Upload button.

- It will compile again, then begin uploading to the board.

- If it gets stuck on 'Connecting...': Double-press the RST button on the board while it is trying to connect.

- When it says 'Done uploading': The micro-ROS firmware is now running on your ESP32.

micro-ROS publisher firmware is now running on your Arduino Nano ESP32.

What is happening now: The ESP32 is trying to connect to a micro-ROS agent over USB serial. Since we have not set up the agent yet (that lives on the Jetson), the ESP32 will keep trying and its LED may blink to indicate it is waiting. This is expected.

# 8. Quick Verification (Without Jetson)

You can't fully test micro-ROS without a ROS 2 machine running the agent. But you can verify the firmware is running:

## Step 8.1 - Check Serial Output

- In Arduino IDE, click Tools > Serial Monitor (or press Ctrl+Shift+M).

- Set the baud rate to 115200 (dropdown in the top-right of the Serial Monitor panel).

- You may see garbled text or nothing. This is normal because the serial port is being used for micro-ROS transport, not debug output.

- Close the Serial Monitor when done (it interferes with micro-ROS transport).

## Step 8.2 - Optional: Test the Agent with Docker on Windows

If you want to verify micro-ROS communication now (without the Jetson), you can run the micro-ROS agent in Docker on your Windows machine:

- Install Docker Desktop: Download from https://www.docker.com/products/docker-desktop/

- After installing Docker Desktop, open a Command Prompt or PowerShell window.

- Run this command (replace COM5 with your actual port):

docker run -it --rm -v /dev:/dev --privileged --net=host microros/micro-ros-agent:humble serial --dev COM5 -v6

Docker on Windows may not directly pass COM ports. If this does not work, that is fine - you will test properly when the Jetson is set up. This step is entirely optional.

## Step 8.3 - How to Know It Will Work Later

If the compile and upload in Section 7 succeeded without errors, your ESP32 firmware is correct. When you later connect it to the Jetson running the micro-ROS agent, the agent will discover the ESP32 node and you will see the published topic.

# 9. Understanding What You Just Installed

Here is a plain-English explanation of every piece:

| Component | What It Does |
| --- | --- |
| Arduino IDE | A program on your PC. You write code in it, click Upload, and it sends the compiled firmware to the board over USB. Think of it as Microsoft Word, but for microcontroller code. |
| ESP32 Board Package | A plug-in for Arduino IDE that teaches it how to compile C/C++ code for the ESP32-S3 chip specifically. Without this, Arduino IDE only knows about basic Arduino boards. |
| micro_ros_arduino | A library (collection of pre-written code) that implements the micro-ROS communication protocol. It lets your ESP32 create ROS 2 nodes, publishers, and subscribers, and talk to a ROS 2 network over USB serial. |
| esp32s3 folder copy | The library ships with pre-built binaries for the standard ESP32 chip. Your Arduino Nano ESP32 uses the newer ESP32-S3. By copying the esp32 folder to esp32s3, you tell the library to use those same binaries for your chip. |
| micro-ROS Agent | A program that runs on the Jetson (or any ROS 2 computer). It bridges between the ESP32's serial stream and the full ROS 2 DDS network. The ESP32 talks to the agent; the agent relays messages to other ROS 2 nodes. |
| XRCE-DDS | The actual protocol used over the USB serial cable. It stands for 'DDS for eXtremely Resource Constrained Environments.' It is a lightweight version of the standard ROS 2 communication protocol (DDS), designed for microcontrollers. |

# 10. Next Steps

You have now completed the ESP32 side of the setup. Here is what comes next:

- Set up the Jetson Orin Nano: Install Ubuntu 22.04 + ROS 2 Humble + the micro-ROS agent.

- Connect ESP32 to Jetson via USB: Run the micro-ROS agent on the Jetson. It will detect the ESP32 node.

- Write the motor control firmware: Replace the publisher example with your stepper motor control code that subscribes to /cmd_pose and drives the 3 L298N motor drivers.

- Implement the scan orchestrator: Write the Jetson-side ROS 2 node that sends 72 pose commands and captures images.

- Integrate AADL model: Load the system architecture model into OSATE 2 for formal verification.

## 10.1 Quick Reference: File Locations on Your PC

| What | Path on Windows 11 |
| --- | --- |
| Arduino IDE | C:\Program Files\Arduino IDE\ |
| Your sketches | C:\Users\YOUR_NAME\Documents\Arduino\ |
| micro_ros_arduino library | C:\Users\YOUR_NAME\Documents\Arduino\libraries\micro_ros_arduino\ |
| esp32s3 fix folder | ...\micro_ros_arduino\src\esp32s3\ |
| ESP32 board tools | C:\Users\YOUR_NAME\AppData\Local\Arduino15\packages\esp32\ |

## 10.2 Troubleshooting Quick Reference

| Problem | Solution |
| --- | --- |
| No COM port in Device Manager | Try different cable. Try different USB port. Double-press RST. |
| 'python: command not found' during compile | Install Python 3 from python.org. Check 'Add to PATH' during install. |
| 'ModuleNotFoundError: pyserial' | Open CMD, run: pip install pyserial |
| 'micro_ros' library not found | Step 6.3: copy esp32 folder to esp32s3 inside micro_ros_arduino/src/ |
| Upload stuck on 'Connecting...' | Double-press RST button on board during upload attempt. |
| Compile fails with board package errors | Use ESP32 board package version 2.0.x, not 3.x. |